BODILOG Project Overview

Project Name: BodiLog Urine Analysis Mobile App
Platform: Cross-platform (iOS & Android)
Software Framework: Flutter/Dart
Software Functions: AI-powered urine test strip analysis with IoT integration

Core Component: Urine Test Chamber detailed in US patent "System and method for urine analysis and personal health monitoring" (https://patents.google.com/patent/US20190073763A1/en?oq=US+patent+2019%2f0073763A1)

Additional Components: healthcare test devices with IoT integration using Bluetooth connection, including:

- Smart scales (body fat percentage, muscle mass, BMI, water weight, and bone density)

- Blood pressure monitors

- Wearable sensors (sleep, and motion tremor for Parkingson’s disease)

- Thermometers

- Oximeters

Prompt 1: Project Setup & Architecture

Create a Flutter project structure for a urine analysis mobile app with the following specifications:

- Flutter version: 3.x (latest stable)

- Dart SDK: 3.x

- Required dependencies:

- camera: ^0.10.5+9 (for image capture)

- image: ^4.1.3 (for image processing)

- path_provider: ^2.1.1 (for local storage)

- http: ^1.1.0 (for cloud upload)

- flutter_blue_plus: ^1.20.0 (for Bluetooth IoT connectivity)

- shared_preferences: ^2.2.2 (for user settings)

- permission_handler: ^11.0.1 (for runtime permissions)

Project structure:

/lib

/models

- test_strip_model.dart

- user_data_model.dart

- iot_device_model.dart

/services

- image_processing_service.dart

- color_analysis_service.dart

- bluetooth_service.dart

- cloud_storage_service.dart

- local_storage_service.dart

/screens

- home_screen.dart

- camera_capture_screen.dart

- results_screen.dart

- dashboard_screen.dart

- settings_screen.dart

/widgets

- test_strip_overlay.dart

- color_calibration_widget.dart

- result_card.dart

/utils

- constants.dart

- image_cropper.dart

- color_matcher.dart

main.dart

Generate the complete pubspec.yaml and main.dart with proper initialization.

Prompt 2: Image Capture Module

Implement a Flutter camera capture screen for urine test strip analysis with these requirements:

UI Requirements:

- Full-screen camera preview with 4:3 aspect ratio

- Overlay guide showing test chamber alignment (rectangle frame)

- Capture button with haptic feedback

- Flash toggle (auto/on/off)

- Focus tap-to-focus functionality

Technical Requirements:

- Resolution: minimum 1920x1080, preferred 4K

- Auto-focus with macro mode support

- Exposure lock after focus

- Image format: JPEG with maximum quality

- Save to temporary directory for processing

Special Features:

- Real-time brightness detection (warn if too dark/too bright)

- Level indicator (gyroscope) to ensure phone is flat

- Countdown timer (3-2-1) before capture to stabilize

The captured image will have:

- Black background

- Fixed layout: barcode (top), RGB color bar (left), test strip (center-right), lookup matrix (bottom)

Generate the complete camera_capture_screen.dart with all dependencies and helper methods.

Prompt 3: Image Segmentation & Processing Service

Create an image processing service in Flutter that segments the captured urine test image into 4 sub-images based on fixed pixel coordinates:

Input: Full captured image (JPEG, variable resolution but consistent aspect ratio)

Processing Steps:

1. Load image and normalize to reference resolution (1080x1920)

2. Extract 4 sub-images using fixed pixel boundaries:

Sub-image 1 (Barcode):

- Coordinates: (10, 10) to (920, 120)

- Purpose: Decode manufacturer, product model, batch number

- Processing: Convert to grayscale, apply barcode decoding (use google_mlkit_barcode_scanning)

Sub-image 2 (RGB Color Bar):

- Coordinates: (20, 130) to (120, 900)

- Purpose: Color calibration

- Contents: 3 color squares (top to bottom):

* Blue: hex #0000FF

* Green: hex #00FF00

* Red: hex #FF0000

- Processing: Extract average RGB values from each square center (20x20 pixels) to create calibration matrix

Sub-image 3 (Test Strip):

- Coordinates: (130, 130) to (920, 900)

- Purpose: Extract 10 test pad colors

- Processing:

* Divide vertically into 10 equal regions (top to bottom)

* For each region, extract center 50x50 pixel average color

* Apply color calibration from Sub-image 2

* Map to test parameters (standard 10 measurements) in order:

1. Leukocytes

2. Nitrite

3. Urobilinogen

4. Protein

5. pH

6. Haemoglobin

7. Specific gravity

8. Ketone

9. Bilirubin

10. Glucose

Sub-image 4 (Lookup Matrix):

- Coordinates: Calculate based on image dimensions (10x7 grid, 70 color squares)

- Purpose: Reference color mapping

- Processing: Extract each square's RGB value into 10x7 matrix

Output:

- BarcodeData object (manufacturer, model, batch, expiry)

- CalibrationData object (RGB correction factors)

- List<TestPad> with 10 items (parameter name, detected color, calibrated RGB)

- LookupMatrix object (10x7 grid of reference colors)

Generate image_processing_service.dart with all data models and processing logic.

Prompt 4: Color Analysis & Result Interpretation

Implement a color analysis service that matches detected test pad colors to standardized urine test results:

Input:

- List of 10 test pad RGB values (from image processing)

- 10x7 lookup matrix RGB values (reference colors)

Analysis Logic for each test pad:

1. Color Distance Calculation:

- Use CIE Delta E 2000 algorithm for perceptual color difference

- Compare test pad color against all 7 colors in its corresponding lookup matrix row

2. Result Determination:

- Find minimum Delta E (closest match)

- If Delta E < 5.0: confident match

- If 5.0 <= Delta E < 10.0: uncertain match (flag for user review)

- If Delta E >= 10.0: no match (error)

3. Value Mapping (example for standard 10-parameter strip):

| Row | Parameter | Default | Test Results |  |  |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Leukocytes | -1 | 0.009 | 70 | 125 | 500 | null | null | WBC/ul |
| 2 | Nitrite | -1 | 0.009 | 999 | 999 | 999 | null | null |  |
| 3 | Urobilinogen | 0.1 | 1 | null | 2 | 4 | 8 | null | mg/dL |
| 4 | Protein | -1 | 0.009 | 30 | 100 | 300 | 1000 | null | mg/dL |
| 5 | pH | 5.0 | 6.0 | 6.5 | 7.0 | 7.5 | 8 | 8.5 |  |
| 6 | Haemoglobin | -1 | 0.009 | 25 | 80 | 200 | null | null |  |
| 7 | Specific Gravity | 1.000 | 1.005 | 1.010 | 1.015 | 1.020 | 1.025 | 1.030 |  |
| 8 | Ketone | -1 | 5 | 15 | 40 | 80 | 160 | null | mg/dL |
| 9 | Bilirubin | -1 | null | 9 | 99 | 999 | null | null |  |
| 10 | Glucose | -1 | 100 | 250 | 500 | 1000 | 2000 | null | mg/dL |

4. Health Interpretation:

- Flag abnormal values with color coding:

* Green: Normal

* Yellow: Borderline (monitor)

* Red: Abnormal (consult doctor)

- Provide brief explanation for each abnormal result

Output: TestResult object with:

- List<TestPadResult> (parameter, value, unit, status, confidence)

- Overall health score (0-100)

- Recommendations list

- Timestamp

- Image thumbnail

Create a dashboard to show the trends of health status

Generate color_analysis_service.dart with Delta E implementation and health interpretation logic.

Prompt 5: Bluetooth IoT Integration

Implement a Bluetooth Low Energy (BLE) service for connecting to IoT health devices:

Supported Devices:

- Smart scales (body fat percentage, muscle mass, BMI, water weight, and bone density)

- Blood pressure monitors

- Wearable sensors (sleep, and motion tremor for Parkingson’s disease)

- Thermometers

- Oximeters

Requirements:

1. Device Discovery:

- Scan for BLE devices with specific service UUIDs

- Filter by known manufacturer IDs

- Display device list with signal strength

- Auto-connect to previously paired devices

2. Connection Management:

- Maintain persistent connection during data collection

- Handle disconnections gracefully with auto-retry

- Support multiple simultaneous connections (up to 3 devices)

3. Data Collection:

- Subscribe to characteristic notifications

- Parse binary data packets according to device protocols

- Validate checksums and data integrity

- Convert raw data to standardized health metrics

4. Data Synchronization:

- Timestamp all readings with device clock sync

- Queue data if app is backgrounded

- Batch upload when connection restored

5. Google Drive Integration

- Connect to Google Drive via user’s login credentials

- Exports health data (test results, devices, readings) as JSON to a "BodiLog Health Data" folder

6. Security:

- Encrypt all BLE communications (BLE 4.2+)

- Require user pairing confirmation

- Store device credentials securely (Flutter Secure Storage)

- User authentication and account profile administration

Generate bluetooth_service.dart with:

- Abstract Device